Table 1 - Test sites characteristics.

| Thesis | Total Trees |  |  |  |  | Removal Trees |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n ha-1 | G ha-1 | V ha-1 | DBH | Hg | n ha-1 | G ha-1 | V ha-1 | DBH | Hg |
|  |  | m2 | m3 | cm | m |  | m2 | m3 | cm | m |
| A | 1033 | 71.89 | 664.446 | 29.8 | 22.59 | 233 | 12.74 | 116.106 | 26.3 | 22.06 |
| B | 875 | 69.62 | 628.501 | 31.7 | 22.86 | 275 | 19.28 | 159.323 | 30.0 | 22.63 |
| C | 704 | 61.98 | 572.519 | 33.4 | 23.77 | 304 | 24.17 | 218.394 | 31.6 | 22.86 |
| n: Stand density; G: basal area; V: stand volume; DBH: diameter at breast height. |  |  |  |  |  |  |  |  |  |  |

Table 2 - Description of work elements and independent variables.

| Work and time elements | Definition | Independent variables |
| --- | --- | --- |
| Felling |  |  |
| Walk to tree - Wtt | Begins when the worker starts toward the tree to be cut and ends when he reaches to the tree. | Distance between trees - Dt Diameter at breast height - DBH |
| Cut - Ct | Begins when the worker starts to make a wedge-shaped notch in the base of the tree to ensure that it accurately faces the felling direction and ends when the worker starts backcut. |  |
| Fell - Ft | Begins when the worker starts cutting the opposite side of the direction of fall and ends when the tree hits the ground. |  |
| Cross-cutting - CRc | Begins when the worker moves to the felled tree and ends when the feller finishes the last of the cross-cutting and starts toward the next tree to be felled. |  |
| Refuel and sharpening | - |  |
| Extraction cycle with cable winch |  |  |
| Travel unloaded – T U | Begins when the farm tractor leaves the landing area and ends when the farm tractor stops in the stump area. | Winching distance - Wd Skidding Distance – Sd Number of logs – Nl Volume - V |
| Hooking - H | Begins when the worker has just grabbed the cable and sets the choker on the tree about 0.5–1.0 m away from the tree end, and ends when the farm tractor operator starts winching. |  |
| Winching - W | Begins when the driver starts to winch and ends when the tree has arrived at the rear part of the farm tractor. |  |
| Travel loaded – T L | Begins when the farm tractor moves to the landing and ends when it reaches the landing. |  |
| Unhooking - Un | Begins when the farm tractor reaches the landing and ends when the load is unhooked. |  |
| Delays – D T | Personal delay, Technical delay, and Operational delay. |  |

Table 3 - Descriptive statistics at site A, B and C for felling phase.

| Parameter | Unit | Site A |  | Site B |  | Site C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean | SD | Mean | SD | Mean | SD |
| Total Cycle Time | min | 7.67 | 2.65 | 8.96 | 3.43 | 9.53 | 3.62 |
| Walk to tree - Wtt | min | 0.91 | 0.34 | 0.89 | 0.34 | 1.03 | 0.39 |
| Cut - Ct | min | 2.49 | 2.60 | 3.30 | 3.42 | 3.48 | 3.60 |
| Fell - Ft | min | 1.29 | 0.12 | 1.54 | 0.16 | 1.70 | 0.17 |
| Cross-cutting - CRc | min | 2.04 | 0.22 | 2.18 | 0.23 | 2.48 | 0.27 |
| Refuel and sharpening – R & S | min | 0.50 | 0.13 | 0.53 | 0.15 | 0.45 | 0.15 |
| Delay T | min | 0.45 | 0.15 | 0.52 | 0.14 | 0.40 | 0.09 |
| Distance | m | 42.98 | 14.78 | 40.83 | 14.04 | 45.87 | 15.77 |
| Diameter | cm | 26.25 | 1.39 | 30.03 | 1.44 | 31.60 | 1.44 |
| Productivity | m3 h-1 | 4.85 | 1.28 | 5.71 | 1.61 | 5.85 | 1.62 |

Table 4 - Descriptive statistics at site A, B and C for skidding phase.

| Parameter | Unit | Site A |  | Site B |  | Site C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean | SD | Mean | SD | Mean | SD |
| Total cycle time | min | 11.01 | 0.91 | 11.30 | 0.68 | 11.42 | 0.84 |
| Time travel unloaded – T U | min | 1.65 | 0.47 | 1.48 | 0.41 | 1.33 | 0.37 |
| Hooking - H | min | 1.44 | 0.39 | 1.53 | 0.41 | 1.61 | 0.48 |
| Winching - W | min | 1.93 | 0.27 | 2.06 | 0.35 | 2.15 | 0.39 |
| Time travel loaded – T L | min | 4.68 | 0.47 | 5.14 | 0.55 | 5.20 | 0.59 |
| Unhooking - Un | min | 0.93 | 0.16 | 1.08 | 0.22 | 1.11 | 0.29 |
| Delay Time – D T | min | 0.38 | 0.21 | 0.50 | 0.19 | 0.53 | 0.26 |
| Bunching distance | m | 37.57 | 10.31 | 33.25 | 9.12 | 30.93 | 40.10 |
| Skidding distance | m | 223.16 | 22.61 | 203.87 | 20.66 | 201.12 | 20.38 |
| Number of logs | - | 4.63 | 0.49 | 4.55 | 0.50 | 4.58 | 0.50 |
| Volume | m3 | 0.56 | 0.06 | 0.73 | 0.08 | 0.78 | 0.09 |
| Productivity | m3 h-1 | 3.04 | 0.42 | 3.89 | 0.54 | 4.11 | 0.52 |

Table 5  -  Productivity equation models for cutting and winching phases.

| Model | Equation |  | F | P | R2adjusted |
| --- | --- | --- | --- | --- | --- |
| Productivity | Felling | P = 7.286 + 0.5 * Diam - 0.452 * TCT - 0.004 * Dist + 0.856 * SS | 856.97 | 0.000 | 0.950 |
|  | Skidding | P = 1.932 - 0.003 * BDist - 0.008 * SDist + 5.339 * V - 0.162 * SS | 342.371 | 0.000 | 0.920 |
| P = Productivity; Diam = Diameter; TCT = Total Cycle Time; Dist = Distance; SS = Study Site (0 =A, 1 = B, 2 = C); BDist = Bunching Distance; SDist = Skidding Distance. |  |  |  |  |  |