Table 1 - Description of study sites.

| Characteristics | Unit | Site A | Site B |
| --- | --- | --- | --- |
| Location | - | Brognaturo | Brognaturo |
| Harvesting method | - | Cut-to-length | Cut-to-length |
| Dominant species | - | Chestnut | Silver fir |
| Forest type | - | High forest | Natural forest |
| Felling equipment | - | Chainsaw | Chainsaw |
| Extraction equipment | - | Farm tractor + Winch | Farm tractor + Grapple |
| Average altitude | m a.s.l. | 1050 | 1100 |
| Stand density | trees/ha | 870 | 570 |
| Stock volume | m3/ha | 948 | 889 |
| Number of trees felled | trees/ha | 261 | 86 |
| Average DBH | cm | 35 | 39 |
| Average height | m | 24 | 25 |
| Average volume per tree | m3 | 1.09 | 1.53 |
| Average slope | % | 30 | 29 |
| Roughness | - | I | I |
| Total area | ha | 8 | 16 |
| Extraction intensity | m3/ha | 284 | 134 |
| Total volume extracted | m3 | 2276 | 2147 |

Table 2 - General characteristics of wood extraction with winches and grapple.

| Machine attachment | General characteristics |
| --- | --- |
| Winch | Use a cable and choker to pull one or more trees to a tractor. The skidding winch is normally attached to the 3-point hitch and takes its power from the tractor PTO. Advantages: Low to medium cost. Suited to a wide range of tractors and sites. When using winches in difficult terrain the load can be dropped and tractor can move to more favourable terrain and winch the log from a distance. Disadvantages: Limited application in thinnings for high density of trees and low accessibility with consequent difficulty of logs motion especially long logs. Skidding often produces dirty logs, which can cause difficulties at the processing stage. Can contribute to both soil and residual tree damage. |
| Grapple | Large hydraulic grapples mounted on the 3-point hitch can be used equally well for transporting cut-to-length logs or tree-length logs. The operator reverses up to the logs or timber stack and ‘grapples’ the load, which can then be hydraulically lifted for transportation. Advantages: Relatively inexpensive. Shortwood can be extracted clean. Operator does not need to leave the cab. Disadvantages: Requires good presentation of material and does not have the flexibility and versatility of typical winch skidders. Needs good sites, detailed planning and site layout is required especially in thinnings. |

Table 3 - Specifications of the machinery used in the two study sites.

| Parameters | Unit | Chainsaw | Tractor | Winch | Grapple |
| --- | --- | --- | --- | --- | --- |
| Producer | - | Husqvarna | Same | Schwarz | Krpan |
| Model | - | 560 XP | Silver 110 | EGV 105 AHK | KL 2200 |
| Power | kW | 3.5 | 81 | - | - |
| Weight | kg | 5.9 | 4700 | - | - |
| Displacement | cm3 | 59.8 | 6000 | - | - |
| Overall length/ width | mm | - | 4250/2735 | - | - |
| Bar length | cm | 40 | - | - | - |
| Minimum power required | kW | - | - | 74 | 40-90 |
| Diameter | mm | - | - | 13 | - |
| Drum capacity | m | - | - | 180 | - |
| Nominal pulling force of winch | kN | - | - | 100 | - |
| Closing force | kN | - | - | - | 70 |
| Min / max opening width | cm | - | - | - | 10/220 |
| Rotation angle on both sides | degrees | - | - | - | ± 40 |
| Load capacity | kg | - | - | - | 3000 |

Table 4 - Elements of work time.

|  |  | Action | Description |
| --- | --- | --- | --- |
| Felling and processing with chainsaw |  | Moving | Begins when the chainsaw operator (CHO) or assistant (AS) starts walking toward the working place and ends when the worker reaches the working place |
|  |  | Felling | Begins when the CHO reaches the tree and ends after the tree is felled on the ground. |
|  |  | Supporting felling | Begins when the AS reaches the tree and ends after the tree is felled on the ground. |
|  |  | Clearing | Cutting and crosscutting the undergrowth |
|  |  | Delimbing | Cutting the branches from the felled tree |
|  |  | Refuel and sharpening | The chain is sharpened every time the fuel chainsaw is filled |
| Extraction with cable winch or grapple |  | Travel unloaded (similar for cable winch and grapple) | Begins when the skidder leaves the landing area and ends when the skidder stops in the stump area |
|  |  | Release and hooking (cable winch) | Begins when the worker has just grabbed the cable and sets the choker on the tree about 0.5–1.0 m away from the tree end, and ends when the skidder operator starts winching |
|  |  | Winching (cable winch) | Begins when the driver starts to winch and ends when the tree has arrived at the rear part of the skidder |
|  |  | Grappling (grapple) | Begins when the grapple of the skidder opens and takes the trees and ends when the grapple is closed |
|  |  | Travel loaded (similar for cable winch and grapple) | Begins when the machine moves to the landing and ends when it reaches the landing |
|  |  | Unhooking (similar for cable winch and grapple) | Begins when the machine reaches the landing and ends when the load is unhooked |
| Delay Time | In both phases (felling and extraction) was considered also miscellaneous time that is not related to productive work time (phone calls, etc.). |  |  |

Table 5 - Assumed cost parameters for machine rate calculation.

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Chainsaw + 2 operators | Tractor +winch + 3 operators | Tractor + grapple+ 2 operators |  |
| Purchase price | € | 980 | 60000 |  | 60000 |
| Salvage value | € | 0 | 12000 |  | 12000 |
| Estimated life | year | 1.2 | 10 |  | 10 |
| Scheduled machine hour | h | 1680 | 1050 |  | 980 |
| Fuel and lubricant | € h-1 | 3.05 | 14.95 |  | 16.08 |
| Annual depreciation | € year-1 | 817 | 4800 |  | 4800 |
| Interest | € year-1 | 36 | 1152 |  | 1152 |
| Total fixed cost | € h-1 | 0.51 | 8.59 |  | 9.21 |
| Total variable cost | € h-1 | 3.54 | 18.15 |  | 19.51 |
| Total labor cost | € h-1 | 42 | 63 |  | 42 |
| Total hourly cost | € h-1 | 46.05 | 89.74 |  | 70.72 |

Table 6 - Productivity equation for sites A and B with manual chainsaw felling and processing.

| Site | Model | Equation | F | P | R2adjusted |
| --- | --- | --- | --- | --- | --- |
| A+B | Productivity | P (m3 h−1) = - 11.427 + 0.369 × DBH (cm) + 0,262 × H (m) | 595.828 | 0.00 | 0.697 |

DBH = diameter at breast height, H = height.

Table 7 - Basic descriptive statistics of operational variables and performance metrics.

| Average parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Site A | Site B |

| Productive machine hour (PMH) | m3 | 2.91 | 5.92 |
| --- | --- | --- | --- |
| Scheduled machine hour (SMH) | m3 | 2.87 | 5.73 |
| Logs extracted per cycle | n | 3 | 2 |
| Skidding distance | m | 276 | 105 |
| Bunching distance | m | 33 | - |
| Volume extracted per cycle | m3 | 0.7 | 0.59 |
| Extraction cost (PMH) | € m-3 | 30.8 | 11.9 |

Table 8 - Time consumption (mean value and standard deviation (SD)) for working cycle elements.

| Work phase |  | Unit | Site A |  | Site B |  | Percent of total time (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean | (SD) | Mean | (SD) | Site A | Site B |
| Felling | Moving | min | 0,40 | 0,17 | 0,52 | 0,19 | 4 | 5 |
|  | Felling | min | 0,61 | 0,25 | 0,78 | 0,29 | 7 | 8 |
|  | Delimbing | min | 6,59 | 0,91 | 7,63 | 0,97 | 77 | 77 |
|  | Refuel/Sharpening | min | 0,49 | 0,19 | 0,52 | 0,21 | 6 | 5 |
|  | Delay time | min | 0,47 | 0,18 | 0,49 | 0,20 | 6 | 5 |
|  | Cycle time | min | 8,56 | 0,88 | 9,93 | 0,94 | 100 | 100 |
| Extraction | Travel unloaded | min | 2.59 | 0.37 | 2.86 | 1.15 | 19 | 39 |
|  | Hooking/Grappling | min | 1.93 | 0.32 | 0.16 | 0.04 | 14 | 2 |
|  | Winching | min | 2.22 | 0.23 | - | - | 16 | - |
|  | Travel loaded | min | 5.76 | 0.41 | 3.05 | 1.11 | 42 | 42 |
|  | Unhooking | min | 1.10 | 0.06 | 1.08 | 0.04 | 8 | 15 |
|  | Delay time | min | 0.21 | 0.06 | 0.16 | 0.03 | 1 | 2 |
|  | Cycle time | min | 13.81 | 0.84 | 7.31 | 2.28 | 100 | 100 |

Table 9 - Cycle time and productivity equations for sites A (cable winch) and B (grapple).

| Site | Model | Equation | F | P | R2adjusted |
| --- | --- | --- | --- | --- | --- |
| A | Cycle time | Equation (1) Ct (min) = 5.817 + 0.039 × Wd (m) + 0.019 × Sd (m) + 0.461 × Nl (n) | 49.327 | 0.00 | 0.788 |
| B | Cycle time | Equation (2) Ct (min) = - 4.233 + 0.110 × Sd (m) | 1028.965 | 0.00 | 0.963 |
| A+B | Productivity | Equation (3) P (m3 h−1) = 12.669 − 0.055 × Sd (m) + 8.024 × V (m3) – 5.771 × St | 28.178 | 0.00 | 0.508 |

Wd = Winching distance, Sd = Skidding Distance, Nl = number of logs, V = Volume, St = Skidding type (0 = winch; 1 = grapple)